Triggerbox

OUT D

OUT C

OUT B

OUT A

INPUT RA5

JMP B

JMP A

SWITCHRA2

+3.3V

GND

GND

| JMP A | JMP B | Frequency [Hz] |
| --- | --- | --- |
| shorted | shorted | 30 |
| open | shorted | 60 |
| shorted | open | 90 |
| open | open | 120 |

/*

* File:   TriggerBox.c

* Author: stru_ro

*

* Date: 2022-03-11 13:48

*/

#pragma config FOSC = INTRCIO   // Oscillator Selection bits (INTOSCIO oscillator: I/O function on RA4/OSC2/CLKOUT pin, I/O function on RA5/OSC1/CLKIN)

#pragma config WDTE = OFF       // Watchdog Timer Enable bit (WDT disabled)

#pragma config PWRTE = OFF      // Power-up Timer Enable bit (PWRT disabled)

#pragma config MCLRE = ON       // MCLR Pin Function Select bit (MCLR pin function is MCLR)

#pragma config CP = OFF         // Code Protection bit (Program memory code protection is disabled)

#pragma config CPD = OFF        // Data Code Protection bit (Data memory code protection is disabled)

#pragma config BOREN = ON       // Brown-out Reset Selection bits (BOR enabled)

#pragma config IESO = OFF        // Internal External Switchover bit (Internal External Switchover mode is enabled)

#pragma config FCMEN = OFF      // Fail-Safe Clock Monitor Enabled bit (Fail-Safe Clock Monitor is disabled)

#include <xc.h>

#include <stdbool.h>

#define _XTAL_FREQ 4000000

const unsigned short FREQ_LUT[4] = {0xBEE5, 0xDF72, 0xEA4C, 0xEFB9};  //timer1 values for 30Hz, 60Hz, 90Hz, 120Hz

unsigned short timer_load_value;

unsigned char freq, pulse, dummy;

void main(void)

{

// Set interrupt enable bits

GIE = 1;   // Global Interrupt Enable Bit

RABIE = 1; // PORTA/PORTB Change Interrupt Enable bit

PEIE = 1;  // Peripherals Interrupt Enable Bit

TMR1IE = 1;   //enable timer1

IOCA2 = 1;  //enable interrupt on change of RA2 pin

IOCA5 = 1;  //enable interrupt on change of RA5 pin

T1CON = 0x00; //turn off Timer1

ANSEL = 0;  //configure I/O pins as 'digital function', not 'analog input'

ANSELH = 0; //configure I/O pins as 'digital function', not 'analog input'

TRISA0 = 1; //configure RA0 as input

TRISA1 = 1; //configure RA1 as input

TRISA2 = 1; //configure RA2 as input

TRISA5 = 1; //configure RA5 as input

TRISB4 = 0; //configure RB4 as output

TRISB5 = 0; //configure RB5 as output

TRISB6 = 0; //configure RB6 as output

TRISB7 = 0; //configure RB7 as output

TRISC7 = 0; //for testing purposes

nRABPU = 0;  //pull-ups are enabled by individual port latches

WPUA0 = 1;  //enable RA0 weak pull-up

WPUA1 = 1;  //enable RA1 weak pull-up

WPUA2 = 1;  //enable RA2 weak pull-up

//pull-up of RA5 is not enabled. RA5 is configured as an input for a signal from the event-camera

timer_load_value = FREQ_LUT[0];

TMR1 = timer_load_value; //set initial timer1 load value

pulse = 0;

if(RA2 == 0) T1CON = 0x01;   //if RA2==0 initially (switch is already closed), then turn on timer1

if(RA5 == 1) T1CON = 0x01;   //if RA5==1 initially (input is already high), then turn on timer1

while(1)

{

freq = RA1 << 1;

freq |= RA0;

timer_load_value = FREQ_LUT[freq];

}

}

void interrupt ISR(void)

{

if(TMR1IF)

{

pulse ^= 0x1;   //toggle pulse

if (pulse==0x0) PORTB = 0x00;

if (pulse==0x1) PORTB = 0xFF;

TMR1 = timer_load_value; //set timer1 load value

TMR1IF = 0; //clear timer1 interrupt

}

if(RABIF)

{

//read PORTA to end the mismatch condition

dummy = PORTA;

__delay_ms(10);   //wait 10ms until switch-bouncing is finished

//read RA2 and RA5 again after delay

if((RA2 == 0) || (RA5 == 1)){

T1CON = 0x01;   //turn on timer1

TMR1 = timer_load_value; //set timer1 load value

}else{

T1CON = 0x00;  //turn off timer1

TMR1IF = 0; //clear timer1 interrupt

PORTB = 0x00;

}

RABIF = 0;  //clear IOC interrupt flag

}

}